All symbols must be specified and a description must be added stating the type of device with its normal state (NO, NC).

No. 1: Design a PLC ladder logic program to control the operation of a conveyor-storage system using the following sequence.

- Program a count-up to keep track of the parts brought into the storage room by the CONVEYOR_IN and detected with a PROXIMITY_IN sensor.

- Program a count-down to keep track of the parts taken out of the storage room by the CONVEYOR_OUT and detected with a PROXIMITY_OUT sensor.

- Both conveyor motors are operated by one START/STOP seal-in pushbutton station.

- A GREEN light must be on if the part count in the storage room is equal to zero (0).

- A YELLOW light must be on if the part count in the storage room is between one (1) and ten (10) (inclusive).

- A RED light must flash on and off in periods of 0.5 seconds if the part count in the storage room is greater than ten (10).

- A RESET pushbutton is used to reset the count.

- Program CONVEYOR_IN and CONVEYOR_OUT using bits 0 and 1 of the output module.

- Program the GREEN, YELLOW, and RED lights using bits 2, 3, and 4 of the output module.

- Program the START and STOP buttons as normally open in bits 0 and 1 of the input module.

- Program PROXIMITY_IN and PROXIMITY_OUT using bits 2 and 3 of the input module.

- Program the RESET pushbutton using bit 4 of the input module.