All symbols must be specified and a description must be added stating the type of device with its normal state (NO, NC).

No. 2: Design a PLC ladder logic program to control the operation of an automatic mixing process using the following sequence:

- A solenoid valve, VALVE_IN, automatically allows fluid in the mixing tank 3 seconds after a level switch, LEVEL_EMPTY, detects when the tank is empty.

- VALVE_IN is automatically de-energized as soon as a level switch, LEVEL_FULL, detects when the tank is full.

- A mixer MOTOR is automatically energized for 10 seconds after the tank is full.

- After the 10-second mixing period the mixer MOTOR is automatically shut down.

- A second solenoid valve, VALVE_OUT, is energized to drain the tank immediately after the MOTOR shuts down.

- VALVE_OUT is automatically de-energized as soon as the level switch, LEVEL_EMPTY, detects when the tank is empty. The filling process must start over at this point.

- A master switch, ON_OFF, is used to manually turn off the solenoid valves and the mixer.

- Hint: Latch/Unlatch both valves and motor with intermediate bits.

- Program ON_OFF on bit 0 of the input module.

- Program LEVEL_EMPTY on bit 1 of the input module.

- Program LEVEL_FULL on bit 2 of the input module.

- Program MOTOR on bit 0 of the output module.

- Program VALVE_IN on bit 1 of the output module.

- Program VALVE_OUT on bit 2 of the output module.